Configure azure via JSON template

https://github.com/Azure/azure-quickstart-templates/tree/master/201-site-to-site-vpn

After Site 2 Site connection is deployed review your Azure gateway address and your Local gateway IP address

Configure the Fortigate

Login into the forgate mangament

Under VPN => IPsecWizard

Select Custom

Configure the VPN tunnel as outlined below

Under Network => Static Routes 
Create a new static route to the Azure vnet address space

Under Policy & Objects => Addresses

Add the Azure vnet address space

Add the Local Address space for the FortiGate

Under Policy & Objects => IPV4 Policy

Allow the firewall to accept incoming traffic from the Azure vnet

Create a 2nd firewall policy to allow outgoing traffic from the FortiGate to the Azure vnet

Under Monitor => IPSec Monitor

Right click to bring up the gateway

Ensure the VPN tunnel comes up on the Fortinet:

The Azure portal will update within a few moments